CSE 332: Computer Architecture & Organization

Assignment#1 Spring 2025

Submitted by,

Touhid Ara Himu
Student ID: 2312187642

Section: 1
Serial: 35

Submitted to,

Dr. Mohammad Abdul Qayum [MAQm]

Submission Date – Sunday, 18 February, 2025

Due Date – Sunday, 18 February, 2025

Contents

1. BJT, MOSFET	8

Definition:	8

Elaboration:	8

Suitable Method:	8

2. Diode and LED (Light Emitting Diode)	8

Definition:	8

Elaboration:	8

Suitable Method:	8

Figures and Flowcharts:	9

3. pMOS, nMOS, CMOS	9

Definition:	9

Elaboration:	9

Figures and Flowcharts:	9

4. Universal Logic Gates	9

Definition:	9

Elaboration:	9

Figures and Flowcharts:	9

5. CMOS 3-input NAND Logic	10

Definition:	10

Figures and Flowcharts:	10

6. Process Node Technology	10

Definition:	10

Figures and Flowcharts:	10

7. Edge-Triggered D Flip-Flop	10

Definition:	10

Figures and Flowcharts:	10

8. Clock Edge	10

Definition:	10

Figures and Flowcharts:	10

9. Combinational Logic	11

Definition:	11

Elaboration:	11

Figures and Flowcharts:	11

10. Sequential Logic	11

Definition:	11

Elaboration:	11

Figures and Flowcharts:	11

11. Moore State Machine	11

Definition:	11

Figures and Flowcharts:	11

12. Register Transfer Level (RTL)	12

Definition:	12

Figures and Flowcharts:	12

13. Timing Diagram	12

Definition:	12

Figures and Flowcharts:	12

14. Karnaugh Map	12

Definition:	12

Figures and Flowcharts:	12

15. Multiplexer (MUX)	12

Definition:	12

Figures and Flowcharts:	12

16. Demultiplexer (DEMUX)	13

Definition:	13

Figures and Flowcharts:	13

17. Decoder	13

Definition:	13

Figures and Flowcharts:	13

18. Encoder	13

Definition:	13

Figures and Flowcharts:	13

19. Full Bit Adder	13

Definition:	13

Figures and Flowcharts:	13

20. Ripple Carry Adder	13

Definition:	13

Figures and Flowcharts:	14

21. Comparator	14

Definition:	14

Figures and Flowcharts:	14

22. Counter	14

Definition:	14

Figures and Flowcharts:	14

23. Shift Register	14

Definition:	14

Figures and Flowcharts:	14

24. Ring Oscillator	14

Definition:	14

Figures and Flowcharts:	14

25. Clock Divider	15

Definition:	15

Figures and Flowcharts:	15

26. Asynchronous Design	15

Definition:	15

Figures and Flowcharts:	15

27. Setup Time	15

Definition:	15

Figures and Flowcharts:	15

28. Hold Time	15

Definition:	15

Figures and Flowcharts:	15

29. Propagation Delay	15

Definition:	15

Figures and Flowcharts:	16

30. Fan-out	16

Definition:	16

Figures and Flowcharts:	16

31. Photolithography in chip fabrication	16

Definition:	16

Elaboration:	16

Figures and Flowcharts:	16

32. RTL Synthesis	16

Definition:	16

Elaboration:	17

Figures and Flowcharts:	17

33. HDL (Hardware Description Language)	17

Definition:	17

Elaboration:	17

Figures and Flowcharts:	17

34. EDA (Electronic Design Automation) Example	17

Definition:	17

Elaboration:	17

Figures and Flowcharts:	18

35. VHDL, Verilog	18

Definition:	18

Elaboration:	18

Figures and Flowcharts:	18

36. FPGA (Field-Programmable Gate Array)	18

Definition:	18

Elaboration:	18

Figures and Flowcharts:	19

37. CPLD (Complex Programmable Logic Device)	19

Definition:	19

Elaboration:	19

Figures and Flowcharts:	19

38. ASIC (Application-Specific Integrated Circuit)	19

Definition:	19

Elaboration:	19

Figures and Flowcharts:	19

39. Gate-Level Design	19

Definition:	19

Elaboration:	20

Figures and Flowcharts:	20

40. Testbench	20

Definition:	20

Elaboration:	20

Figures and Flowcharts:	20

41. DFT (Design for Testability)	20

Definition:	20

Elaboration:	20

Figures and Flowcharts:	20

42. Logic Synthesis	21

Definition:	21

Elaboration:	21

Figures and Flowcharts:	21

43. Power Optimization	21

Definition:	21

Elaboration:	21

Figures and Flowcharts:	21

44. Heterogeneous Multicore Processor	21

Definition:	21

Elaboration:	22

Figures and Flowcharts:	22

45. Cache	22

Definition:	22

Elaboration:	22

Figures and Flowcharts:	22

46. Main Memory	22

Definition:	22

Elaboration:	22

Figures and Flowcharts:	22

47. Five Steps from Sand to Chip	23

Figures and Flowcharts:	23

48. Five Stages from HDL to Chip Layout	23

Figures and Flowcharts:	23

49. RISC, CISC	23

Definition:	23

Elaboration:	23

Figures and Flowcharts:	23

50. Wafer, Die, Chip Packaging	24

Definition:	24

Elaboration:	24

Figures and Flowcharts:	24

## 1. BJT, MOSFET

### Definition:

- BJT(Bipolar Junction Transistor) is a current-controlled device that consists of three layers: Emitter, Base, and Collector. It operates in active, cutoff, or saturation regions.

- MOSFET(Metal-Oxide-Semiconductor Field-Effect Transistor) is a voltage-controlled device that comes in two types: n-channel and p-channel. It is widely used in digital circuits due to its high efficiency and switching speed.

### Elaboration:

- BJT works based on the movement of charge carriers (electrons and holes) between its three terminals.

- MOSFETs operate by controlling a channel between the source and drain using a gate voltage.

### Suitable Method:

- BJTs are preferred in analog circuits, whereas MOSFETs are widely used in digital circuits due to their lower power consumption.

## 2. Diode and LED (Light Emitting Diode)

### Definition:

- A Diode is a two-terminal semiconductor device that allows current to flow in only one direction.

- A Light Emitting Diode (LED) is a diode that emits light when forward-biased.

### Elaboration:

- The diode functions based on the PN junction principle and is used in rectifiers, clamping circuits, etc.

- LEDs are used in indicators, displays, and lighting due to their efficiency.

### Suitable Method:

- Rectifiers use diodes to convert AC to DC.

### Figures and Flowcharts:

- Diode and LED Structure

## 3. pMOS, nMOS, CMOS

### Definition:

- pMOS (Positive-channel MOSFET): Uses holes as charge carriers.

- nMOS (Negative-channel MOSFET): Uses electrons as charge carriers.

- CMOS (Complementary MOS): Uses both pMOS and nMOS transistors to create energy-efficient logic circuits.

### Elaboration:

- CMOS technology is widely used in modern VLSI circuits due to low power dissipation.

### Figures and Flowcharts:

- CMOS Inverter Diagram

## 4. Universal Logic Gates

### Definition:

- Universal logic gates are NAND and NOR gates that can be used to implement any Boolean function.

### Elaboration:

- NAND and NOR gates are fundamental for digital circuit design as they allow for complete logical operations.

### Figures and Flowcharts:

- Universal Gate Diagrams

## 5. CMOS 3-input NAND Logic

### Definition:

- A CMOS NAND gate with 3 inputs uses both pMOS and nMOS transistors to implement logic functions.

### Figures and Flowcharts:

- CMOS NAND Gate Implementation

## 6. Process Node Technology

### Definition:

- The process node represents the manufacturing technology used in semiconductor fabrication, denoted in nanometers (e.g., 7nm, 5nm).

### Figures and Flowcharts:

- Semiconductor Process Node Evolution

## 7. Edge-Triggered D Flip-Flop

### Definition:

- A D Flip-Flop is a sequential circuit that captures the input value at a specific clock edge (rising or falling).

### Figures and Flowcharts:

- D Flip-Flop Timing Diagram

## 8. Clock Edge

### Definition:

- The transition of a clock signal (rising or falling) that triggers sequential logic operations.

### Figures and Flowcharts:

- Clock Edge Timing

## 9. Combinational Logic

### Definition:

- Combinational logic is a type of digital logic where the output is determined solely by the current inputs without any memory elements.

### Elaboration:

- It is used in arithmetic operations, multiplexers, decoders, and other logic circuits.

### Figures and Flowcharts:

- Combinational Logic Diagram

## 10. Sequential Logic

### Definition:

- Sequential logic is a type of digital logic where the output depends on both current inputs and past inputs (memory).

### Elaboration:

- It includes flip-flops, counters, and shift registers, used in clocked circuits.

### Figures and Flowcharts:

- Sequential Logic Diagram

## 11. Moore State Machine

### Definition:

- A Moore State Machine is a finite-state machine where the outputs depend only on the current state.

### Figures and Flowcharts:

- Moore FSM Diagram

## 12. Register Transfer Level (RTL)

### Definition:

- RTL is a design abstraction that describes how data is transferred between registers using combinational logic.

### Figures and Flowcharts:

- RTL Design Flow

## 13. Timing Diagram

### Definition:

- A graphical representation of signals in a digital circuit over time.

### Figures and Flowcharts:

- Timing Diagram Example

## 14. Karnaugh Map

### Definition:

- A visual method for simplifying Boolean expressions.

### Figures and Flowcharts:

- K-Map Examples

## 15. Multiplexer (MUX)

### Definition:

- A circuit that selects one of many inputs and forwards it to the output based on select lines.

### Figures and Flowcharts:

- MUX Circuit Diagram

## 16. Demultiplexer (DEMUX)

### Definition:

- A circuit that distributes a single input to multiple outputs based on select lines.

### Figures and Flowcharts:

- DEMUX Circuit Diagram

## 17. Decoder

### Definition:

- A circuit that converts an n-bit input into a unique 2^n-bit output.

### Figures and Flowcharts:

- Decoder Circuit

## 18. Encoder

### Definition:

- A circuit that converts 2^n inputs into an n-bit output.

### Figures and Flowcharts:

- Encoder Circuit

## 19. Full Bit Adder

### Definition:

- A digital circuit that adds three input bits and outputs a sum and carry.

### Figures and Flowcharts:

- Full Adder Circuit

## 20. Ripple Carry Adder

### Definition:

- A type of binary adder that chains multiple full adders in sequence.

### Figures and Flowcharts:

- Ripple Carry Adder Diagram

## 21. Comparator

### Definition:

- A digital circuit that compares two binary numbers and outputs the comparison result (equal, greater, or lesser).

### Figures and Flowcharts:

- Comparator Circuit

## 22. Counter

### Definition:

- A sequential circuit that counts pulses and stores the count value.

### Figures and Flowcharts:

- Counter Circuit

## 23. Shift Register

### Definition:

- A sequential circuit that shifts binary data in a serial or parallel manner.

### Figures and Flowcharts:

- Shift Register Diagram

## 24. Ring Oscillator

### Definition:

- A simple oscillator circuit made of an odd number of inverters.

### Figures and Flowcharts:

- Ring Oscillator Diagram

## 25. Clock Divider

### Definition:

- A circuit that reduces the frequency of a clock signal by an integer factor.

### Figures and Flowcharts:

- Clock Divider Circuit

## 26. Asynchronous Design

### Definition:

- A circuit design methodology that does not rely on a global clock signal.

### Figures and Flowcharts:

- Asynchronous Logic

## 27. Setup Time

### Definition:

- The minimum time before a clock edge when the input must remain stable.

### Figures and Flowcharts:

- Setup Time Diagram

## 28. Hold Time

### Definition:

- The minimum time after a clock edge when the input must remain stable.

### Figures and Flowcharts:

- Hold Time Diagram

## 29. Propagation Delay

### Definition:

- The time taken for a signal to travel from input to output.

### Figures and Flowcharts:

- Propagation Delay Analysis

## 30. Fan-out

### Definition:

- The maximum number of logic inputs a gate can drive without degradation.

### Figures and Flowcharts:

- Fan-out Concept

## 31. Photolithography in chip fabrication

### Definition:

- Photolithography is a process used in semiconductor manufacturing to transfer intricate circuit patterns onto silicon wafers using light.

### Elaboration:

- A photoresist is applied to the wafer and exposed to ultraviolet (UV) light through a mask containing the circuit design.

- The exposed regions of the photoresist are either hardened (positive resist) or dissolved (negative resist), allowing selective etching of the underlying material.

- Photolithography enables the fabrication of nanoscale transistor structures and is crucial for advanced process nodes like 5nm and below.

### Figures and Flowcharts:

- Photolithography Process

- Semiconductor Lithography Overview

## 32. RTL Synthesis

### Definition:

- RTL (Register Transfer Level) Synthesis is the process of converting high-level hardware descriptions (written in HDL) into an optimized gate-level representation that can be implemented in hardware.

### Elaboration:

- RTL synthesis takes an HDL design (e.g., Verilog or VHDL) and translates it into logic gates and flip-flops using Boolean optimization techniques.

- The synthesized netlist is then mapped to a specific FPGA or ASIC technology for fabrication.

- Key steps include logic synthesis, technology mapping, and gate-level optimization to ensure area, power, and speed efficiency.

### Figures and Flowcharts:

- RTL Synthesis Flow

- Synthesis Process Overview

## 33. HDL (Hardware Description Language)

### Definition:

- HDL (Hardware Description Language) is a specialized programming language used to describe and model the behavior and structure of digital circuits.

### Elaboration:

- Common HDLs include VHDL and Verilog, which allow designers to specify circuit behavior at a high level before synthesis into logic gates.

- HDLs support concurrent execution and are used for both simulation and hardware implementation in ASICs and FPGAs.

- They enable hardware prototyping, verification, and automated logic synthesis.

### Figures and Flowcharts:

- HDL Workflow

- Verilog vs VHDL Comparison

## 34. EDA (Electronic Design Automation) Example

### Definition:

- Electronic Design Automation (EDA) refers to software tools used to design, simulate, and verify integrated circuits (ICs) and electronic systems.

### Elaboration:

- EDA tools assist in circuit design, logic synthesis, timing analysis, and physical layout.

- Examples include:

  - Synopsys Design Compiler – for logic synthesis.

  - Cadence Virtuoso – for IC layout and simulation.

  - Xilinx Vivado – for FPGA design and implementation.

- These tools optimize power, area, and timing constraints for ASICs and FPGAs.

### Figures and Flowcharts:

- EDA Tool Workflow

- VLSI Design Flow Using EDA

Let me know if you need further elaboration!

## 35. VHDL, Verilog

### Definition:

- VHDL (VHSIC Hardware Description Language) and Verilog are two popular Hardware Description Languages (HDLs) used to model digital circuits.

- VHDL is strongly typed and more verbose, whereas Verilog is more concise and similar to C.

### Elaboration:

- VHDL is widely used in defense and aerospace industries due to its strong typing.

- Verilog is preferred in commercial ASIC and FPGA design due to its simplicity.

### Figures and Flowcharts:

- VHDL vs. Verilog Comparison

## 36. FPGA (Field-Programmable Gate Array)

### Definition:

- An FPGA is a reconfigurable digital circuit consisting of an array of programmable logic blocks connected via configurable interconnects.

### Elaboration:

- Unlike ASICs, FPGAs can be reprogrammed after manufacturing, making them ideal for prototyping and low-volume applications.

### Figures and Flowcharts:

- FPGA Architecture

## 37. CPLD (Complex Programmable Logic Device)

### Definition:

- CPLDs are programmable logic devices that contain multiple macrocells and offer non-volatile storage for configuration.

### Elaboration:

- They provide faster and more predictable timing than FPGAs but are less flexible in terms of logic resources.

### Figures and Flowcharts:

- CPLD vs. FPGA

## 38. ASIC (Application-Specific Integrated Circuit)

### Definition:

- ASIC is a custom-designed integrated circuit optimized for a specific application, offering high performance and low power consumption.

### Elaboration:

- ASICs are commonly used in mobile processors, GPUs, and networking hardware.

- Unlike FPGAs, ASICs cannot be reprogrammed once manufactured.

### Figures and Flowcharts:

- ASIC Design Process

## 39. Gate-Level Design

### Definition:

- A gate-level design represents digital circuits in terms of basic logic gates and their interconnections.

### Elaboration:

- It is the lowest level of abstraction before hardware implementation, generated after RTL synthesis.

### Figures and Flowcharts:

- Gate-Level Schematic

## 40. Testbench

### Definition:

- A testbench is a simulation environment used to verify the correctness of an HDL-based circuit by applying test vectors.

### Elaboration:

- Testbenches include stimulus generation, response checking, and timing analysis.

### Figures and Flowcharts:

- Testbench Simulation

## 41. DFT (Design for Testability)

### Definition:

- Design for Testability (DFT) refers to design techniques that improve the ability to test an integrated circuit (IC) for defects after fabrication.

### Elaboration:

- DFT incorporates structures like scan chains, Built-In Self-Test (BIST), and boundary scan (JTAG) to enhance fault detection.

- It ensures that testing procedures, such as Automatic Test Pattern Generation (ATPG), can efficiently diagnose faults in digital circuits.

### Figures and Flowcharts:

- DFT Implementation Flow

- Scan Chain Example

## 42. Logic Synthesis

### Definition:

- Logic Synthesis is the process of converting an abstract hardware description (HDL) into an optimized gate-level netlist.

### Elaboration:

- It involves Boolean optimization, technology mapping, and gate-level transformation to minimize area, power, and timing constraints.

- The synthesized netlist is used for placement and routing in the physical design phase.

### Figures and Flowcharts:

- Logic Synthesis Flow

- Optimization Example

## 43. Power Optimization

### Definition:

- Power optimization refers to design techniques used to reduce power consumption in digital circuits.

### Elaboration:

- Methods include clock gating, power gating, dynamic voltage and frequency scaling (DVFS), and multi-Vt optimization.

- Essential in battery-powered and high-performance computing systems to enhance energy efficiency.

### Figures and Flowcharts:

- Power Optimization Techniques

- Clock Gating Implementation

## 44. Heterogeneous Multicore Processor

### Definition:

- A Heterogeneous Multicore Processor consists of multiple cores with different architectures optimized for various tasks.

### Elaboration:

- Examples include ARM big.LITTLE architecture, where high-performance cores handle intensive tasks, and power-efficient cores handle background processes.

- Used in smartphones, embedded systems, and AI accelerators.

### Figures and Flowcharts:

- Heterogeneous Multicore Architecture

- Big.LITTLE Concept

## 45. Cache

### Definition:

- Cache is a small, high-speed memory located between the processor and main memory to store frequently accessed data.

### Elaboration:

- Cache is categorized into L1, L2, and L3 levels, where L1 is the fastest and smallest.

- Uses techniques like associativity, replacement policies, and prefetching to enhance CPU performance.

### Figures and Flowcharts:

- Cache Memory Hierarchy

- Cache Replacement Policies

## 46. Main Memory

### Definition:

- Main memory (RAM) is the primary storage medium that holds data and programs for immediate access by the CPU.

### Elaboration:

- Types include SRAM (Static RAM) and DRAM (Dynamic RAM).

- Main memory acts as a bridge between slow storage (HDD/SSD) and fast CPU cache.

### Figures and Flowcharts:

- Memory Hierarchy

- RAM Architecture

## 47. Five Steps from Sand to Chip

- Purification – Silicon is extracted from sand and purified.

- Crystal Growth – A pure silicon ingot is grown using the Czochralski process.

- Wafer Formation – The ingot is sliced into thin wafers.

- Photolithography & Etching – Circuits are patterned onto the wafer.

- Doping & Metallization – Transistors and interconnects are formed.

### Figures and Flowcharts:

- Silicon Chip Manufacturing

## 48. Five Stages from HDL to Chip Layout

- RTL Design – Write hardware description in Verilog/VHDL.

- Logic Synthesis – Convert RTL to gate-level representation.

- Placement & Routing – Physically arrange the gates on a chip.

- Timing Analysis & Optimization – Ensure timing constraints are met.

- Fabrication & Packaging – Manufacture and package the chip.

### Figures and Flowcharts:

- VLSI Design Flow

## 49. RISC, CISC

### Definition:

- RISC (Reduced Instruction Set Computing) uses simple instructions executed in a single cycle.

- CISC (Complex Instruction Set Computing) uses complex instructions that may take multiple cycles.

### Elaboration:

- RISC is used in ARM processors for power efficiency.

- CISC is used in x86 processors for compatibility and performance.

### Figures and Flowcharts:

- RISC vs. CISC Comparison

## 50. Wafer, Die, Chip Packaging

### Definition:

- Wafer – A thin slice of silicon that contains multiple dies.

- Die – A single functional unit cut from the wafer.

- Chip Packaging – The final step where the die is enclosed in a protective casing.

### Elaboration:

- Wafers undergo photolithography, doping, and etching before being diced into individual dies.

- Each die is tested, packaged, and integrated into PCBs or embedded systems.

### Figures and Flowcharts:

- Wafer-to-Chip Process

- Chip Packaging Techniques