# CSE332: Computer Architecture and Organization

Assignment#5

Submitted by,

Name: Muhtadina Serniabat Tasin

# Student ID: 2232160642

Submitted to,
Faculty Initial: MAQm
Section: 5

Submission Date:

Spring 2025

North South University

# Modified MIPS Architecture for JAL, JR, SLL, and SRL Instructions

# MIPS Architecture

# Datapath Modifications

1. JAL instruction:

2. JR instruction:

3. SLL/SRL instructions:

Fig – SLL instruction

Fig – SRL Instruction

# Control Signal Table

| Instruction | RegDst | ALUSrc | MemToReg | RegWrite | MemRead | MemWrite | Branch | Jump | ALU OP[1] | ALU OP[0] | Special Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| JAL | 1 | x | 0 | 1 | 0 | 0 | 0 | 1 | x | x | Write PC+4 to $ra |
| JR | x | x | x | 0 | 0 | 0 | 0 | 1 | x | x | PC ← $ra |
| SLL | 1 | 0 | 0 | 1 | 0 | 0 | 0 | 0 | 1 | 0 | Shift Left |
| SRL | 1 | 0 | 0 | 1 | 0 | 0 | 0 | 0 | 1 | 0 | Shift Right |

# Modified control.v

// modified file: control.v `timescale 1ns/1ns module Control_unit(input [5:0] Opcode, input [5:0] Func, input Zero, output reg MemtoReg, output reg MemWrite, output reg ALUSrc, output reg RegDst, output reg RegWrite, output reg Jump, output PCSrc, output reg [3:0] ALUControl, output reg JAL, // New signal for JAL output reg JR // New signal for JR ); reg [7:0] temp; reg Branch,BNE; always @(*) begin // Default values JAL = 1'b0; JR = 1'b0; case (Opcode) 6'b000000: begin // R-type temp <= 8'b11000000; case (Func) 6'b100000: ALUControl <= 4'b0000; // ADD 6'b100001: ALUControl <= 4'b0000; // ADDU 6'b100010: ALUControl <= 4'b0001; // SUB 6'b100011: ALUControl <= 4'b0001; // SUBU 6'b100100: ALUControl <= 4'b0010; // AND 6'b100101: ALUControl <= 4'b0011; // OR 6'b100110: ALUControl <= 4'b0100; // XOR 6'b100111: ALUControl <= 4'b1010; // NOR 6'b101010: ALUControl <= 4'b1000; // SLT 6'b101011: ALUControl <= 4'b1001; // SLTU 6'b000000: ALUControl <= 4'b0101; // SLL 6'b000010: ALUControl <= 4'b0110; // SRL 6'b000011: ALUControl <= 4'b0111; // SRA 6'b000100: ALUControl <= 4'b1011; // SLLV 6'b000110: ALUControl <= 4'b1100; // SRLV 6'b000111: ALUControl <= 4'b1101; // SRAV 6'b001000: begin // JR temp <= 8'b00000010; ALUControl <= 4'b0000; JR <= 1'b1; end endcase end 6'b100011: begin // LW temp <= 8'b10100100; ALUControl <= 4'b0000; end 6'b101011: begin // SW temp <= 8'b00101000; ALUControl <= 4'b0000; end 6'b000100: begin // BEQ temp <= 8'b00010000; ALUControl <= 4'b0001; end 6'b000101: begin // BNE temp <= 8'b00010001; ALUControl <= 4'b0001; end 6'b001000: begin // ADDI temp <= 8'b10100000; ALUControl <= 4'b0000; end 6'b001001: begin // ADDIU temp <= 8'b10100000; ALUControl <= 4'b0000; end 6'b001100: begin // ANDI temp <= 8'b10100000; ALUControl <= 4'b0010; end 6'b001101: begin // ORI temp <= 8'b10100000; ALUControl <= 4'b0011; end 6'b001110: begin // XORI temp <= 8'b10100000; ALUControl <= 4'b0100; end 6'b001010: begin // SLTI temp <= 8'b10100000; ALUControl <= 4'b1000; end 6'b001011: begin // SLTIU temp <= 8'b10100000; ALUControl <= 4'b1001; end 6'b000010: begin // J temp <= 8'b00000010; ALUControl <= 4'b0010; end 6'b000011: begin // JAL temp <= 8'b11000010; ALUControl <= 4'b0010; JAL <= 1'b1; end 6'b001111: begin // LUI temp <= 8'b10100000; ALUControl <= 4'b1110; end default: temp <= 8'bxxxxxxxx; // NOP endcase {RegWrite,RegDst,ALUSrc,Branch,MemWrite,MemtoReg,Jump,BNE} = temp; end assign PCSrc = Branch & (Zero ^ BNE); endmodule