# CSE 332: Computer Architecture and Organization

Assignment#2 Spring 2025

Submitted by,

Muhtadina Serniabat Tasin
ID: 2232160642

Course: CSE332
Section: 5
Semester: Spring 2025

Submitted to,
Dr. Mohammad Abdul Qayum [MAQm]

Submission Date: 1 March, 2025

Due Date: 4 March, 2025

1. MIPS instructions:

- Move content of register r11 into register r23.

- Load register r15 with the value 265,140.

- Store the value 55,259 into a word at memory location 40000.

- The equation a = b + c, assuming a, b and c are integer variables at memory location 0b10000000, 0b1001000 and 0b100010011100, respectively.

- The equation y[5] = y[10] + x, assuming x corresponds to register r2, and y[1] is an integer at the memory location 524,292 is the first element of vector y.

- The statement: if (a <= b) go to lab1, assuming that variable a corresponds to the word at memory address 400, and variable b corresponds to register r10. The instruction with lab1 is before the branch instruction and there are 20 (machine) instructions between those two instructions.

Ans. 1.

move $r23, $r11 # Moving content of Register r11 into Register r23.

Ans. 2.

Value = 265,140

So, (265140)10 = (40BB4)16

Upper 16 bits = (0 × 0004)16 = (4)10

Lower 16 bits = (0 × 0BB4)16 = (2996)10

lui $r15, 4 # Load Upper Immediate: (4 * 2^16 = 262144) ori $r15, $r15, 2996 # OR Immediate: Add the lower 16-bit part # (265140 - 262144 = 2996)

Ans. 3.

| lui $t0, 0 ori $t0, $t0, 40000 | # Load upper part of memory address # Complete the address |
| --- | --- |
| li $t1, 55259 sw $t1, 0($t0) | # Load value 55,259 into a temporary register # Store value at address 40,000 |

Ans. 4. 
variable a at location = 0b10000000 = (128)10
variable b at location = 0b1001000 = (72)10
variable c at location = 0b100010011100 = (1100)10

lw $t0, 72($zero) # Load b from memory (address 72) into $t0 lw $t1, 1100($zero) # Load c from memory (address 1100) into $t1 add $t2, $t0, $t1 # t2 = b + c sw $t2, 128($zero) # Store result in a (address 128)

Ans. 5. 
y[1] at location = 524292
So, y[i] at = 52492 + (i-1) × 4

y[5] = 524292 + 16 = 524308
y[10] = 524292 + 36 = 524328

lw $t0, 524328($zero) # Load y[10] from memory at 524,328 add $t0, $t0, $r2 # y[10] + x sw $t0, 524312($zero) # Store result in y[5] at 524,308

Ans. 6.

lw $t0, 400($zero) # Load a from memory ble $t0, $r10, lab1 # Branch if a <= b

2. Bit Pattern for each MIPS Instruction:

lap: lw r16, 0(13) addi r8, r2, -513 nop sw r8, -18(r4) bne r3, r0, lap %lap=25

| Instruction | Type | Opcode | rs | rt | rd/Offset | shamt | funct | Machine Code (Binary) | Machine Code (Hex) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| lw r16, 0(13) | I-Type | 100011 | 01101 | 10000 | 0000 0000 0000 0000 | - | - | 100011 01101 10000 0000000000000000 | 0x8D100000 |
| addi r8, r2, -513 | I-Type | 001000 | 00010 | 01000 | 1111 1110 0000 0000 | - | - | 001000 00010 01000 1111111000000000 | 0x2048FE00 |
| nop (encoded as sll $0, $0, 0) | R-Type | 000000 | 00000 | 00000 | 00000 | 00000 | 000000 | 000000 00000 00000 00000 00000 000000 | 0x00000000 |
| sw r8, -18(r4) | I-Type | 101011 | 00100 | 01000 | 1111 1111 1110 1110 | - | - | 101011 00100 01000 1111111111101110 | 0xA488FFEE |
| bne r3, r0, lap (25) | I-Type | 000101 | 00011 | 00000 | 0000 0000 0001 1001 | - | - | 000101 00011 00000 0000000000011001 | 0x14600019 |

3. Assembly Program in MIPS32 ISA for the following C program:

/// C program #define N 11 int ARRAY[N]= {1, 3, 5, 7, 9, 11, 13, 15, 17, 19, 21}; void main (void) { int total = sum_array(ARRAY); printf ("The total is: %d", total); } int sum_array(int ARRAY[N]) { int total=0; for (int i=0; i<N; i++ ) { total+ =ARRAY[i]; } return total; }

# MIPS32 ISA of the following C program .data ARRAY: .word 1, 3, 5, 7, 9, 11, 13, 15, 17, 19, 21 # Define ARRAY with 11 elements N: .word 11 # Define N = 11 message: .asciiz "The total is: " # String for printf output .text .globl main main: # Load address of ARRAY into $a0 la $a0, ARRAY # Call sum_array function jal sum_array # Store result in $s0 for printing move $s0, $v0 # Print message "The total is: " li $v0, 4 la $a0, message syscall # Print the sum li $v0, 1 move $a0, $s0 syscall # Exit program li $v0, 10 syscall sum_array: # Function prologue addi $sp, $sp, -8 # Allocate space on stack sw $ra, 4($sp) # Save return address sw $s1, 0($sp) # Save s1 li $v0, 0 # total = 0 li $s1, 0 # i = 0 loop: # Check if i < N li $t0, 11 bge $s1, $t0, end_sum_array # Load ARRAY[i] sll $t1, $s1, 2 # i * 4 (word offset) add $t2, $a0, $t1 # Address of ARRAY[i] lw $t3, 0($t2) # Load ARRAY[i] into $t3 # total += ARRAY[i] add $v0, $v0, $t3 # Increment i addi $s1, $s1, 1 j loop end_sum_array: # Restore stack and return lw $ra, 4($sp) lw $s1, 0($sp) addi $sp, $sp, 8 jr $ra