- Clean Verilog code

- Detailed explanation of models used

- Testbench

- Simulation instructions

- Bonus tips for presentation and full marks

## ✅ Part a: 32-bit ALU Verilog Code

// full_adder_en.v - 1-bit ALU

module full_adder_en (

input A, B, Cin,

input Ainvert, Binvert,

input [1:0] Op,

input Less,

output Result, Cout

);

wire Ainput, Binput, sum, andX, orX;

assign Ainput = Ainvert ? ~A : A;

assign Binput = Binvert ? ~B : B;

assign andX = Ainput & Binput;

assign orX = Ainput | Binput;

assign sum = Ainput ^ Binput ^ Cin;

assign Cout = (Ainput & Binput) | (Ainput & Cin) | (Binput & Cin);

mux4x1 m1(sum, andX, orX, Less, Op, Result);

endmodule

// mux4x1.v

module mux4x1 (

input A, B, C, D,

input [1:0] Sel,

output reg Q

);

always @(*) begin

case (Sel)

2'b00: Q = A;

2'b01: Q = B;

2'b10: Q = C;

2'b11: Q = D;

endcase

end

endmodule

// ALU 32-bit - alu32.v

module alu32 (

input [31:0] A, B,

input [3:0] Op,

output [31:0] Result,

output Cout, Overflow

);

wire [31:0] Carry;

wire Set, Less;

wire Ainv = Op[0];

wire Binv = Op[1];

wire [1:0] ALUop = Op[3:2];

wire CoutLast;

full_adder_en FA0(A[0], B[0], Binv, Ainv, Binv, ALUop, Less, Result[0], Carry[0]);

genvar i;

generate

for (i = 1; i < 31; i = i + 1) begin : alu_loop

full_adder_en FA(A[i], B[i], Carry[i-1], Ainv, Binv, ALUop, 1'b0, Result[i], Carry[i]);

end

endgenerate

wire C30 = Carry[30];

full_adder_en FA31(A[31], B[31], Carry[30], Ainv, Binv, ALUop, 1'b0, Result[31], CoutLast);

assign Overflow = C30 ^ CoutLast;

assign Set = Overflow ^ Result[31];

assign Less = Set;

assign Cout = CoutLast;

endmodule

## ✅ Part b: Model Used

### Model Type: Hierarchical + Structural

- Structural: Because the 32-bit ALU is built using 1-bit ALUs (full_adder_en).

- Hierarchical: Because modules (mux4x1, full_adder_en) are instantiated inside other modules (alu32).

### Component Explanation

- full_adder_en: 1-bit ALU that performs operations based on control signals.

- mux4x1: Selects between AND, OR, SUM, SLT.

- Op[3:0]: ALU control signal:

  - Op[0]: Ainvert

  - Op[1]: Binvert

  - Op[3:2]: Operation select (00=ADD, 01=AND, 10=OR, 11=SLT)

### Port Comments

input A, B          // Input operands for 1-bit ALU

input Cin           // Carry input for addition

input Ainvert       // Invert A (for NOT/negative ops)

input Binvert       // Invert B (for subtraction)

input [1:0] Op      // ALU operation selector

input Less          // Used only for SLT

output Result       // Output of the 1-bit ALU

output Cout         // Carry output

## ✅ Part c: ALU Testbench

module test_alu;

reg [31:0] A, B;

reg [3:0] Op;

wire [31:0] Result;

wire Cout, Overflow;

alu32 uut (.A(A), .B(B), .Op(Op), .Result(Result), .Cout(Cout), .Overflow(Overflow));

initial begin

$display("Time\tA\t\t\tB\t\t\tOp\tResult\t\t\tCout Overflow");

$monitor("%0t\t%h\t%h\t%h\t%h\t%b\t%b", $time, A, B, Op, Result, Cout, Overflow);

// AND

A = 32'hF0F0F0F0; B = 32'h0F0F0F0F; Op = 4'b0001; #10;

// OR

A = 32'hF0F0F0F0; B = 32'h0F0F0F0F; Op = 4'b0010; #10;

// ADD

A = 32'd15; B = 32'd10; Op = 4'b0011; #10;

// SUB

A = 32'd20; B = 32'd5; Op = 4'b0111; #10;

// SLT

A = 32'd2; B = 32'd5; Op = 4'b1111; #10;

$stop;

end

endmodule

## ✅ Part d: Simulation in ModelSim

- Create a project in ModelSim.

- Add:

  - alu32.v

  - full_adder_en.v

  - mux4x1.v

  - test_alu.v

- Compile → No errors? Great!

- Simulate:

  - Right-click test_alu → Simulate

  - Add signals to Wave → Run simulation

  - Check if Result, Cout, Overflow are correct.

## ✅ Bonus Tips for 100/100

- ✅ Submit separate .v files (each module).

- ✅ Attach simulation screenshots (waveform output for all 5 test cases).

- ✅ Comment your Verilog code thoroughly.

- ✅ Include a short write-up in your PDF explaining each part (1–2 lines/module).